Y

Product: TMB Plus for HRP, Two Solution Substrate

Catalog Number: TMBP

Lot Number: 20006

Size: A: 100ml; B: 1ml.

Suggested Usage:

Equilibrate both A and B to room temperature.  Mix 1ml of product B with 100ml product A.   Add 50-100ul/well for 96-well ELISA microtiter plate.  Blue color (read at 650nm) will develop and the reaction can be stopped with 50-100ul/well of 1.0M HCl, or 1M H2SO4 and turn into yellow color (read at 450nm).  Discard the unused mixed product and do not use it after 30 minutes from mixing.

Short Term Storage: 2-8oC as a non-mixed liquid for 6 months.

Long Term Storage: -20oC as a non-mixed liquid for up to two year.